# Beach Sand Review Procedures Vary by Locations of Fill Placement and Fill Volumes

| REVIEW PROCEDURES FOR BEACH SAND COMPATIBILITY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fill Location | Volume | Permit Type1 And Reviewer2 | Type Review | Minimum Number of Review Samples Needed |  | QA/QC Plan |
|  |  |  |  | Excavation Material | Native Material |  |
| Beach | ≤200 cy | FP/FI | Physical Samples | 1 sample | 1 sample | No |
|  | >200 cy and <600 cy | FP/PM | Physical Samples | 1 sample | 1 sample | Yes |
|  | ≥600 cy and <2000 cy | AP/PM | Lab reports or mine assays | 2 core borings with 1 sample per soil strata | 1 sample per 500 feet of beach length | Yes |
|  | ≥2000 cy | AP/CE | Lab reports or mine assays | 2 core borings with 1 sample per soil strata | 1 sample per 500 feet of beach length | Yes |
| Dune System | ≤200 cy3 | FP/FI | Physical Samples | 1 sample | Visual inspection | No |
|  | >200 cy and <600 cy | FP/AP/PM | Physical Samples | 1 sample | 1 sample | No |
|  | ≥600 cy and <2000 cy | AP/PM | Lab reports or mine assays | 2 core borings with 1 sample per soil strata | 1 sample per 1000 feet of beach length | Yes |
|  | ≥2000 cy | AP/CE | Lab reports or mine assays | 2 core borings with 1 sample per soil strata | 1 sample per 1000 feet of beach length | Yes |
| Upland of Dune System | ≤200 cy3 | FP/FI | Physical Samples | 1 sample | Visual inspection | No |
|  | >200 cy | FP/AP/PM | Physical Samples | 1 sample | 1 sample | No |
| Between ECL and MHWL | Any Amount | AP/LOC/CE | Lab reports or mine assays | 2 core borings with 1 sample per soil strata | 1 sample per 500 feet of beach length | Yes |
| Seaward of MHWL | Beach sand compatibility review and activity authorization handled by the Joint Coastal Permitting (JCP) Section |  |  |  |  |  |
| 1Permit Type: FP (Field Permit) AP (Administrative Permit) LOC (JCP Letter of Consent) |  |  | 3≤200 cy: For volumes of fill ≤200 cy with placement on or upland of the dune system, the field inspector’s authorization is further limited by the source of fill, as follows: - Source is landward of CCCL, or - Source is upland of the dune system and not resulting in >1-foot lower grade elevation. |  |  |  |
| 2Beach Sand Compatibility Reviewer: FI (Field Inspector) PM (Permit Manager) CE (Coastal Engineering Staff) |  |  |  |  |  |  |

4.6.4. Beach Sand Compatibility Table [09/14/2010 Draft]