Miami River Watershed Event. Field notes, Observations and remarks:

Gerard Philizaire FHMDC Environmental Health, Engineering Division

Station # 101

Site ID : FPL Station.

Issues with garbage containers no bags or without cover some containers with holes at the bottom.

Some location missing containers.

Station # 102

Site ID : Underneath Brickell bridge.

Existing storm sewer covered with dirt, sinking and cover shift to fix

Underneath the bridge dirty with human feces, used mattress, and dead animals.

No trash can around and dog’s pup all around.

Station # 103

Site ID : Building restaurant and Apartment next to the bridge south to Hyatt Hotel.

Storm water over flow and badly smell.

Restaurant manager acknowledges aware of the issue and state it is an existing grease intercepror.

Immediate follow up was made with the DBPR representative for correction. Inspector was called for evaluation.

Station # 104

Site ID : City of Miami Park

Park closed. Observation of trash can on site without bottom and over flow trash can.

Station # 105

Site ID : Fort Dallas Park

Garbage on the ground, user have not regularly used the existing trash can.

Station # 106

Site ID : Miami circle. Florida department of State.

Struggling with maintenance.

Station # 107

Site ID : 99 SW 6 Street. South Miami Place.

Existing open dumpster right at the street side. Garbage can on top of a storm drain.

Station # 112

Site ID : Jose Marti Park Responsible : Miami Dade Transit.

Dog pups all over the grass.Hire private company for garbage pickup and maintenance.no side wall.

Debris of rain water on existing rocks edge.

Station # 113

Site ID : Jose Marti pump station/ Miami Dade water and sewer department.

Garbage can full and no existing trash bag.

Station # 114

Site ID : Sanitary sewer on 342 SW 7 Ave

Many homeless around. Storm sewer clogged and run off to the river.

Station # 115

NW South river Drive

No side wall, garbage and debris all around.Existing  gravity sewer cross line.

Station # 116

Site ID : 114 NW North River Dr. and NW 3 Street .

Giant sewer line 82 inches crossing the River. Side wall collapse, even though doesn’t affect the pipe line.

Nearby park minimum 6 cats seen and existing feeding cat area by visitors.

Station # 117

Site ID : First street Bridge and South river drive

Homeless habitat.

Station # 118

Site ID : 1407 NW 7 Street

Miami Dade County / Public Housing community development

Open Banks. Stagnant water behing.

Station # 119

Site ID :Dead end NW 16 Ave and South shore south river Drive.

Responsible : City of Miami

Yard cleaning it’s a must.

Station # 120

Site ID : Sewell Park

No plug on dumpster and dumpster out of the camouflage cage.

Feeding cats around.

Station # 122

Site ID : Fern Isle Park , 2201 NW South Shore

Ducks. South walk tributary. No maintenance.

Station # 123

Site ID : Miami River Rapids.  Comments N/A

Station # 124

Site ID : Miami Bridge  2810 NW South River Drive

Next to Paradise Trailer Park .Open dumpsters without cover. Dirty and nasty creek.

Neighbors dump trashes to the creek. Stagnant water. Disgusting Trailer Park.

Station # 125

Site ID : 27 Ave South Shore. North walk tributary.

Existing garbage underneath the street bridge.

Station # 126

Site ID : Auto Salvage NW South River Drive.

Open Dumpsters , unpaved area.

Station # 127

Site ID : 3825 NW South River Drive. Super Mix Concrete Plan

Concrete trucks washing on site and water run off to the river no existing drainage.

Water drain to the street with cement residue as well to the river.

Station # 128

Site ID : CSX North River Drive. Rail Road crossing

Trash all along.