# Summary of Coastal Watershed Challenges and Needs in Northwest Florida

The tables below provide an initial representation of the challenges facing coastal watersheds in northwest Florida.  General strategies appropriate for addressing these challenges are also indicated.  The information presented is not necessarily comprehensive and it has not been reviewed by external agencies.  Many of the challenges and strategies listed could be applied generically to most if not all coastal systems.  The information provided below, however, is intended summarize the major challenges and associated strategies specific to each watershed listed.

| Apalachicola River and Bay System Inclusive of Apalachicola Bay, St. Vincent Sound, East Bay, St. George Sound, Alligator Harbor, and tributary rivers and streams |  |
| --- | --- |
| Plans: (1) Apalachicola River and Bay Management Plan (2) Tates Hell State Forest Hydrologic Restoration Plan (3) Local Stormwater Master Plans – City of Apalachicola |  |
| Watershed Challenges | Corresponding Strategies |
| Estuarine freshwater needs | Apalachicola Bay freshwater needs assessment; interstate coordination |
| Fisheries management | Oyster bed management; FFWC and DACS fisheries and shellfish programs |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zone protection |
| Loss of shoreline and littoral habitat due to shoreline alteration and erosion | Shoreline protection and restoration (e.g., riparian buffer zones and shoreline habitat restoration) |
| Inadequate coastal community and rural wastewater management | Sewer line extensions and connection of residences and businesses currently served by septic systems; sewer pit replacements; funding for advanced septic systems and responsible management entities |
| Local outreach and stakeholder coordination | Funding assistance for local watershed initiatives |
| Wetland hydrologic impacts | Tates Hell Swamp hydrologic restoration |
| Coastal and tributary floodplain alteration and habitat loss | Floodplain restoration projects |

| Pensacola Bay System Inclusive of Pensacola, Escambia, Blackwater, and East bays; western Santa Rosa Sound; Big Lagoon; and tributary rivers and streams |  |
| --- | --- |
| Plans: (1) Pensacola Bay System SWIM Plan (2) Local Stormwater Master Plans – Escambia County, City of Pensacola, Gulf Breeze, possibly more |  |
| Watershed Challenges | Corresponding Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zone protection |
| Loss of shoreline and littoral habitat due to shoreline alteration and erosion | Shoreline protection and restoration (e.g., riparian buffer zones and shoreline habitat restoration) |
| Degraded seagrass habitats, Escambia, Pensacola, and East bays; possibly Santa Rosa Sound and Big Lagoon (per DEP report) | Seagrass assessments; water quality improvement projects; urban stormwater retrofit projects; stormwater best management practices; riparian buffer protection, and shoreline restoration |
| Local outreach and stakeholder coordination | Funding assistance for local watershed initiatives |
| Discharge of treated wastewater into surface waters and sprayfields | Advanced wastewater treatment; Reuse of reclaimed water |
| Degraded urban bayou habitats, including littoral habitat loss and sediment degradation and toxicity | Bayou basin plans; best management practices; stormwater retrofits; habitat restoration and sediment remediation |
| Potential marine invasive species | Assessment of the potential for marine invasive species problems, including related to biofouling, commercial vessel ballast, and the adequacy of management practices. Lead agencies anticipated to include FFWC, DACS, federal agencies, and commercial ports. |

| Choctawhatchee River and Bay Watershed Inclusive of Choctawhatchee Bay, eastern Santa Rosa Sound, the Choctawhatchee River, contributing tributary streams, and the coastal dune lakes |  |
| --- | --- |
| Plans: (1) Choctawhatchee River and Bay System SWIM Plan (2) Local Stormwater Master Plans – Fort Walton Beach, Destin, Niceville, Okaloosa County, Walton County, possibly more |  |
| Watershed Challenges | Corresponding Project Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zone protection |
| Loss of shoreline and littoral habitat due to shoreline alteration and erosion | Shoreline protection and restoration (e.g., riparian buffer zones and shoreline habitat restoration) |
| Potentially degraded seagrass habitats | Seagrass assessments; water quality improvement projects; urban stormwater retrofit projects; stormwater best management practices; riparian buffer protection, and shoreline restoration |
| Degraded urban bayou habitats, including littoral habitat loss and sediment degradation and toxicity | Bayou basin plans; best management practices; stormwater retrofits; habitat restoration and sediment remediation |
| Untreated discharges via mosquito control ditches | Wetland and stormwater treatment systems |
| Local outreach and stakeholder coordination | Funding assistance for local watershed initiatives |
| Discharge of treated wastewater into surface waters and sprayfields | Advanced wastewater treatment; Reuse of reclaimed water |
| Degradation of coastal dune lake water and habitat quality | Stormwater treatment systems, riparian buffer zones, and other best management practices |

| St. Andrew Bay Watershed Inclusive of West Bay, St. Andrew Bay, North Bay, East Bay, St. Joseph Bay, Lake Powell, and tributary streams |  |
| --- | --- |
| Plans: (1) St. Andrew Bay Watershed SWIM Plan (2) Local Stormwater Master Plans – St. Andrew Bay Watershed Stormwater Plan (CDM) |  |
| Watershed Challenges | Corresponding Project Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zone protection |
| Loss of shoreline and littoral habitat due to shoreline alteration and erosion | Shoreline protection and restoration (e.g., riparian buffer zones and shoreline habitat restoration) |
| Degraded and lost seagrass habitats | Seagrass assessments; water quality improvement projects; urban stormwater retrofit projects; stormwater best management practices; riparian buffer protection, and shoreline restoration |
| Degraded urban bayou habitats, including littoral habitat loss and sediment degradation and toxicity; includes multiple bayous and Grand Lagoon | Bayou basin plans; best management practices; stormwater retrofits; habitat restoration and sediment remediation |
| Untreated discharges via forestry drainage ditches | Wetland and stormwater treatment systems |
| Local outreach and stakeholder coordination | Funding assistance for local watershed initiatives |
| Direct wastewater discharges into the St. Andrew Bay Estuary | Reuse of reclaimed water |
| Degradation of coastal dune lake water and habitat quality (Lake Powell) | Implementation of stormwater treatment systems, riparian buffer zones, and other best management practices |
| Potential marine invasive species | Assessment of the potential for marine invasive species problems, including related to biofouling, commercial vessel ballast, and the adequacy of management practices. Lead agencies anticipated to include FFWC, DACS, federal agencies, and commercial ports. |

| Ochlockonee River and Bay Watershed Including Ochlockonee Bay, the Ochlockonee River, and tributary streams |  |
| --- | --- |
| Plans: (1) Lake Jackson SWIM Plan (2) Draft Ochlockonee River and Bay SWIM Plan (3) Local Stormwater Master Plans – Leon County, City of Tallahassee, City of Quincy |  |
| Watershed Challenges | Corresponding Project Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zones |
| Inadequate coastal community and rural wastewater management | Sewer line extensions and connection of residences and businesses currently served by septic systems; sewer pit replacements; funding for advanced septic systems and responsible management entities |
| Wetland hydrologic impacts | Tates Hell Swamp hydrologic restoration |

| St. Marks River and Apalachee Bay Watershed Including Apalachee Bay, the St. Marks and Wakulla rivers, and tributary streams |  |
| --- | --- |
| Plans: (1) St. Marks River Watershed SWIM Plan (2) Local Stormwater Master Plans – Leon County, City of Tallahassee, Wakulla County |  |
| Watershed Challenges | Corresponding Project Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices; agricultural best management practices; riparian buffer zone protection |
| Karst region nutrient enrichment affecting Apalachee Bay and its tributaries | Karst effective stormwater and wastewater systems to reduce nutrient and other pollutant loading – research, demonstration projects, and implementation and retrofit projects |
| Disposal of treated wastewater within the Wakulla Karst Plain | Advanced wastewater treatment upgrades for domestic wastewater treatment plants; reuse of reclaimed water |
| Inadequate rural wastewater management | Advanced onsite sewage treatment and disposal systems, connection fee assistance, monitoring, and establishment and funding of responsible management entities |
| High bacteria and nutrient levels in Apalachee Bay | Funding for advanced onsite sewage treatment and disposal systems and karst-effective stormwater and wastewater treatment systems |

| Perdido River and Bay Watershed |  |
| --- | --- |
| Plans: (1) Draft Perdido River and Bay Watershed SWIM Plan (2) Local Stormwater Master Plans – Escambia County, City of Pensacola |  |
| Watershed Challenges | Corresponding Project Strategies |
| Stormwater runoff and nonpoint source pollution | Urban stormwater retrofit projects; stormwater best management practices |
| Needs for protection and restoration of estuarine embayments, including Bayou Garcon, Tarkiln Bay, and others | Bayou basin plans; best management practices; stormwater retrofits; habitat restoration and sediment remediation |
| Loss of shoreline and littoral habitat due to shoreline alteration and erosion | Shoreline protection and restoration (e.g., riparian buffer zones and shoreline habitat restoration) |
| Degraded urban stream systems, resulting in habitat loss, sedimentation, and discharge of poor quality water into Perdido Bay | Stream restoration using natural channel design techniques |
| Discharge of treated wastewater into surface waters and sprayfields | Advanced wastewater treatment; reuse of reclaimed water |
| Overall diminished bay water quality and habitat quality | Reduction of industrial and domestic wastewater discharges; advanced treatment systems; construction of urban stormwater retrofits |

# NWFWMD Plan Links

SWIM Plans – http://www.nwfwmd.state.fl.us/rmd/swim/swimdesc.htm

Tates Hell State Forest Hydrologic Restoration Plan – http://nwfwmdwetlands.com/index.php?Page=30